TOOLBOX TALK Personal Protective Equipment

| Duration | 10 minutes | Audience | All site operatives |
| --- | --- | --- | --- |
| Conducted By | [NAME & POSITION] | Date & Location | [DATE] ¦ [SITE NAME] |

KEY MESSAGE PPE is your last line of defence. It only works if it fits correctly, is in good condition and is actually worn.

INTRODUCTION — READ THIS OUT

PPE — Personal Protective Equipment — is there to protect you when everything else has been done. It is not a substitute for good risk control. But when you need it, you really need it. Today we are going to talk about the PPE required on this site, how to check it is fit for purpose and what to do if it is damaged or does not fit properly.

KEY POINTS TO COVER

- PPE is a legal requirement — The Personal Protective Equipment at Work Regulations 2022 require employers to provide suitable PPE and employees to use it correctly. Refusing to wear PPE on site is a disciplinary matter and can result in removal from site.

- Mandatory PPE on this site — As a minimum on this site: safety helmet, high-visibility clothing, safety footwear with steel toecap and midsole, and gloves for handling materials. Additional PPE will be required for specific tasks — check your RAMS or ask your supervisor.

- Check it before you use it — Inspect your PPE at the start of every day. Check your helmet for cracks — especially inside the shell. Check your harness stitching and buckles. Check your safety boots for sole separation. If in doubt, replace it.

- Fit matters — PPE that does not fit correctly does not protect you. Helmets must be adjusted to fit. Gloves must allow you to grip safely. Respirators must be face-fit tested. If your PPE does not fit, report it to your supervisor and get a replacement.

- Replace it — Helmets must be replaced after any significant impact — even if you cannot see damage. Harnesses involved in a fall must be taken out of service immediately. Do not use PPE past its expiry date or manufacturer's recommended service life.

WHAT TO LOOK FOR ON SITE

| GOOD PRACTICE - All mandatory PPE being worn correctly at all times - Helmets adjusted to fit — chin strap used at height - Gloves appropriate to the task being worn - PPE stored correctly — not left in sunlight or wet conditions - Damaged PPE reported and replaced immediately | WARNING SIGNS - Helmet worn on the back of the head or not at all - Hi-vis vest but no helmet - Gloves removed for comfort on tasks requiring them - Cracked, split or damaged PPE still being used - Safety boots with worn soles or toe caps |
| --- | --- |

DISCUSSION QUESTIONS — ASK THE TEAM

- What PPE is mandatory on this site today and for your specific tasks?

- When did you last check your helmet for damage — including the inside?

- What would you do if a colleague was not wearing their PPE correctly?

- Is there any task today where you think additional PPE might be needed?

KEY TAKEAWAYS — WHAT WE ALL AGREE TO DO

- We will wear all mandatory PPE correctly at all times while on site - We will inspect our PPE at the start of every working day - We will report any damaged or ill-fitting PPE to the supervisor immediately - We will not pressure colleagues to remove PPE for any reason - We will use task-specific PPE as identified in the RAMS for our work

PRE-TASK SAFETY CHECK

Confirm the following before starting work today:

| Check Item | Yes | No | N/A |
| --- | --- | --- | --- |
| All operatives have mandatory PPE — helmet, hi-vis, safety boots, gloves |  |  |  |
| Task-specific PPE identified and available for today's work |  |  |  |
| PPE inspected by each operative before starting |  |  |  |
| Replacement PPE available on site if required |  |  |  |
| All operatives know the site procedure for reporting damaged PPE |  |  |  |

ATTENDANCE REGISTER

By signing below, operatives confirm they have attended this toolbox talk and understood the content.

| Name (Print) | Trade | Signature | Company | Date |
| --- | --- | --- | --- | --- |

Document produced by The Site Office — thesiteoffice.uk — Professional Construction Documents for UK Contractors