TOOLBOX TALK Scaffolding and Access Equipment

| Duration | 10 minutes | Audience | All site operatives working from or near scaffold |
| --- | --- | --- | --- |
| Conducted By | [NAME & POSITION] | Date & Location | [DATE] ¦ [SITE NAME] |

KEY MESSAGE Scaffold is only safe if it has been inspected and signed off. Check the tag before you step on it.

INTRODUCTION — READ THIS OUT

Scaffold failures and falls from scaffold kill and seriously injure construction workers every year. The majority of incidents happen not because scaffold was poorly erected, but because it was altered, overloaded, damaged or used without being inspected. Today we are going to cover what every person on this site needs to check before using scaffold, and what to do if something does not look right.

KEY POINTS TO COVER

- The CISRS inspection tag — Every section of scaffold on this site must have a current CISRS inspection tag. The tag is completed by a competent scaffold inspector after inspection — which must happen every 7 days and after any event that might affect stability such as high winds or an impact. Before stepping onto any scaffold, find the tag and check the date. If there is no tag or it is out of date — do not use the scaffold and report it immediately.

- What to look for — Before using scaffold check: all boards are fully boarded with no gaps, no damaged or rotten boards, guard rails are at least 950mm high, a mid-rail is in place, toe boards are fitted, the scaffold is tied to the building at correct intervals, standards are plumb and not damaged, and access ladders are secure and extend 1 metre above the landing.

- Loading — Scaffold has a maximum load capacity — typically 1.5 kN per square metre for general purpose scaffold. Do not stack excessive materials on scaffold boards. One pack of tiles on a standard scaffold is very different to stacking three packs. If you are unsure about loading, ask the scaffolding contractor before placing materials.

- Never alter scaffold — Under no circumstances should any non-scaffolder alter, dismantle or add to a scaffold. Removing a guard rail to make access easier, removing a tie to open a window, adding an extra board — these all compromise the structural integrity of the scaffold. If a change is needed, contact the scaffold contractor.

- Mobile scaffold towers — PASMA towers must be erected and dismantled only by trained operatives. Castors must be locked before any person ascends. Outriggers or stabilisers must be deployed. Access is by internal ladder only — through the trapdoor. Never move a tower with a person on it. Maximum height-to-base ratio must be observed.

WHAT TO LOOK FOR ON SITE

| GOOD PRACTICE - CISRS inspection tag visible, in date and signed - All boards present, undamaged and fully overlapping standards - Guard rails, mid-rails and toe boards in place on all working lifts - Scaffold properly tied to building structure - Access ladders secured and extending 1m above landing - Materials distributed evenly — not overloaded in one area | WARNING SIGNS - No CISRS tag or tag out of date - Missing boards, gaps in decking or damaged boards - Guard rail removed or tied back - Toe boards missing from working platforms - Scaffold altered by non-scaffolders - Materials piled excessively on one scaffold lift |
| --- | --- |

DISCUSSION QUESTIONS — ASK THE TEAM

- Where is the CISRS inspection tag on the scaffold you will be working from today?

- What would you do if you found a guard rail had been removed on the scaffold?

- Has anyone here ever seen scaffold being altered by a non-scaffolder?

- Who do we contact if we have concerns about the scaffold on this site?

KEY TAKEAWAYS — WHAT WE ALL AGREE TO DO

- We will check the CISRS inspection tag before using any scaffold — every day - We will never alter, dismantle or add to scaffold unless we are a qualified scaffolder - We will report any scaffold defects to the site manager immediately and not use the affected section - We will not overload scaffold platforms with excessive materials - We will use the designated access points — ladders and staircase towers only

PRE-TASK SAFETY CHECK

Confirm the following before starting work today:

| Check Item | Yes | No | N/A |
| --- | --- | --- | --- |
| CISRS inspection tag present and dated within the last 7 days on all scaffold sections |  |  |  |
| All boards present, undamaged and no gaps in decking |  |  |  |
| Guard rails, mid-rails and toe boards in place on all working lifts |  |  |  |
| Scaffold ties to building structure checked — none missing or loose |  |  |  |
| Access ladders secure and extending minimum 1m above landing |  |  |  |
| No evidence of scaffold being altered without scaffolding contractor involvement |  |  |  |

ATTENDANCE REGISTER

By signing below, operatives confirm they have attended this toolbox talk and understood the content.

| Name (Print) | Trade | Signature | Company | Date |
| --- | --- | --- | --- | --- |

Document produced by The Site Office — thesiteoffice.uk — Professional Construction Documents for UK Contractors