TOOLBOX TALK Emergency Procedures

| Duration | 10 minutes | Audience | All site operatives — essential for all new starters |
| --- | --- | --- | --- |
| Conducted By | [NAME & POSITION] | Date & Location | [DATE] ¦ [SITE NAME] |

KEY MESSAGE In an emergency, people do what they have practised. Know the procedures now — not when it is happening.

INTRODUCTION — READ THIS OUT

Emergencies on construction sites happen. Accidents, fires, service strikes, structural failures, medical emergencies. When they happen, the difference between a controlled response and a chaotic one can be the difference between life and death. The time to learn the emergency procedures is not when the alarm is going off. Today we are making sure every person on this site knows exactly what to do.

KEY POINTS TO COVER

- Fire — get out, stay out, call 999 — If you discover a fire — raise the alarm immediately. Shout, activate the nearest fire alarm point. Do not stop to collect tools or belongings. Do not use lifts. Evacuate to the muster point — which is [INSERT MUSTER POINT LOCATION]. Call 999. Do not re-enter the building or site until the fire service declares it safe. Account for all persons at the muster point.

- Serious injury — call 999 first — For any serious injury — loss of consciousness, suspected spinal injury, severe bleeding, suspected heart attack — call 999 immediately. Do not move the casualty unless they are in immediate danger. Administer first aid if trained. Stay with the casualty and keep them as calm and still as possible until the ambulance arrives. The site first aider is [INSERT NAME].

- Gas leak — evacuate and call — If you smell gas or suspect a gas leak — do not operate any electrical switches. Evacuate the area immediately — a minimum 25-metre exclusion zone. Call the National Gas Emergency line: 0800 111 999. Call 999. Ventilate if safe to do so — open doors and windows. Do not re-enter until cleared by the gas emergency service.

- Service strike — If plant or hand tools strike an underground service — stop work immediately. If gas — evacuate 25 metres and call 0800 111 999 and 999. If electric — do not touch the cable or anything in contact with it, call 999 and the DNO. If water — isolate if possible, protect from flooding, notify the site manager. Never attempt to repair a struck service.

- The muster point and roll call — The muster point for this site is [INSERT LOCATION]. On evacuation, all persons must proceed immediately to the muster point. The site manager will carry out a roll call using the site register. Nobody leaves the muster point until accounted for. Nobody re-enters the site until the all-clear is given. Visitors must be accompanied to the muster point.

WHAT TO LOOK FOR ON SITE

| GOOD PRACTICE - Muster point clearly signed and known to all operatives - Fire alarm points identified and accessible - First aider identified and first aid kit available - Emergency contact numbers displayed in welfare cabin - All operatives included in site register for roll call - Emergency procedures covered in site induction | WARNING SIGNS - Operatives unsure of where the muster point is - Fire alarm points blocked by materials - No first aider on site or first aid kit not stocked - Emergency numbers not displayed on site - Site register not being maintained - Emergency procedures not covered in induction |
| --- | --- |

DISCUSSION QUESTIONS — ASK THE TEAM

- Can everyone tell me where the muster point is on this site?

- Who is the site first aider and where is the first aid kit?

- What is the gas emergency number — can everyone remember it?

- When did we last check the first aid kit and fire extinguishers on this site?

KEY TAKEAWAYS — WHAT WE ALL AGREE TO DO

- We all know the muster point location and will go there immediately on any evacuation - We will call 999 first for any serious injury — before anything else - We know the gas emergency number: 0800 111 999 - We will stop work immediately and evacuate if a service is struck - We know who the site first aider is and where the first aid kit is kept

PRE-TASK SAFETY CHECK

Confirm the following before starting work today:

| Check Item | Yes | No | N/A |
| --- | --- | --- | --- |
| Muster point is signed and all operatives know its location |  |  |  |
| Fire alarm call points are identified and accessible |  |  |  |
| First aider is on site and first aid kit is stocked and accessible |  |  |  |
| Emergency contact numbers are displayed in the welfare cabin |  |  |  |
| Site register is up to date and available for roll call |  |  |  |
| All operatives have been briefed on emergency procedures during induction |  |  |  |

ATTENDANCE REGISTER

By signing below, operatives confirm they have attended this toolbox talk and understood the content.

| Name (Print) | Trade | Signature | Company | Date |
| --- | --- | --- | --- | --- |

Document produced by The Site Office — thesiteoffice.uk — Professional Construction Documents for UK Contractors