TOOLBOX TALK Fire Safety on Site

| Duration | 10 minutes | Audience | All site operatives |
| --- | --- | --- | --- |
| Conducted By | [NAME & POSITION] | Date & Location | [DATE] ¦ [SITE NAME] |

KEY MESSAGE Construction sites are high fire risk environments. Know what to do before a fire starts — not after.

INTRODUCTION — READ THIS OUT

Construction sites are one of the highest fire risk environments there is. Combustible materials everywhere, hot works, temporary electrical supplies, fuel storage. A fire can spread incredibly fast on a site — particularly in timber-framed structures or buildings under fit-out. Today we are going to make sure every person on this site knows exactly what to do if a fire starts.

KEY POINTS TO COVER

- Prevention is everything — Most site fires are preventable. Keep combustible waste — timber off-cuts, packaging, shavings — cleared to skips daily. Do not allow combustibles to accumulate in any area. Store fuels and gases correctly — upright, in a ventilated cage, away from ignition sources.

- Hot works — Any hot works — welding, grinding, disc cutting, gas torch — requires a hot works permit. The area must be cleared of combustibles beforehand. A fire extinguisher must be within reach. After completing hot works, a fire watch must be maintained for 30 minutes. Many site fires start from smouldering that begins long after the work is done.

- Know the muster point — The muster point for this site is [INSERT LOCATION]. Every person on site must know where it is. If you are new to this site today — find out before you start work. In the event of a fire alarm, evacuate immediately. Do not collect tools or equipment.

- Extinguisher types — Know which extinguisher to use. CO2 (black label) — electrical fires. Dry powder (blue label) — gas and liquid fires. Water (red) — general materials. Never use water on electrical fires. Fire extinguishers are for small, contained fires only. If in doubt, get out.

- Raise the alarm first — If you discover a fire — raise the alarm first. Shout, activate the alarm, tell others. Then call 999. Only then consider tackling a small fire with an extinguisher — if it is safe to do so. Never put yourself between a fire and your exit route.

WHAT TO LOOK FOR ON SITE

| GOOD PRACTICE - Combustible waste cleared to skips daily - Hot works permit in place before welding or gas torch work - Fire extinguishers accessible and within date - Fire escape routes clear of materials - Gas cylinders stored upright in ventilated cage - Muster point clearly signed and known to all | WARNING SIGNS - Timber off-cuts and waste accumulating in work areas - Hot works without a permit - Fire extinguishers blocked by materials - Fire doors wedged open - Fuel stored near ignition sources - No one knows where the muster point is |
| --- | --- |

DISCUSSION QUESTIONS — ASK THE TEAM

- Where is the muster point on this site? Can everyone confirm they know?

- What is the hot works permit procedure on this site?

- Has anyone here ever had to use a fire extinguisher? What happened?

- What types of fire extinguisher do we have on this site and where are they located?

KEY TAKEAWAYS — WHAT WE ALL AGREE TO DO

- We all know the muster point and will go there immediately if the alarm sounds - We will never prop open fire doors or block fire escape routes with materials - We will obtain a hot works permit before any welding, grinding or gas torch work - We will carry out a 30-minute fire watch after completing all hot works - We will clear combustible waste from our work area daily

PRE-TASK SAFETY CHECK

Confirm the following before starting work today:

| Check Item | Yes | No | N/A |
| --- | --- | --- | --- |
| All operatives know the location of the muster point |  |  |  |
| Fire extinguishers are accessible, signed and within their service date |  |  |  |
| Fire escape routes are clear of materials and obstructions |  |  |  |
| Hot works permit obtained for any welding, cutting or torch work today |  |  |  |
| Combustible waste has been cleared from work areas |  |  |  |
| Gas cylinders stored correctly — upright, caged, ventilated |  |  |  |

ATTENDANCE REGISTER

By signing below, operatives confirm they have attended this toolbox talk and understood the content.

| Name (Print) | Trade | Signature | Company | Date |
| --- | --- | --- | --- | --- |

Document produced by The Site Office — thesiteoffice.uk — Professional Construction Documents for UK Contractors